Log

Thursday, 30 May 2024

08:48

| # Date: 30/05/2024 09:02 | # Topic voltage sensing circuit # Task Research Study Implementation | # Resources High DC voltage sensor module design #E08 ¦ Vaibhav Sugandhi: advanced voltage sensing method # Components The AMC1311 is a versatile and reliable isolated amplifier suitable for a wide range of high-voltage sensing applications. Its high accuracy, isolation capability, and robustness against noise make it an excellent choice for designing voltage sensing circuits in high-voltage systems. ## AMC1311 *AMC1311x High-Impedance, 2-V Input, Reinforced Isolated Amplifiers datasheet (Rev. C) (ti.com) AMC1311 data sheet, product information and support ¦ TI.com ## TL431 #277 Understanding OPTOCOUPLER or OPTO-ISOLATOR (youtube.com) Switching mode power supply #277 Understanding OPTOCOUPLER or OPTO-ISOLATOR Zener Diode 1N4727A # Additional Notes - Safety discharge resistor/ bleeder resistor # Results |  |
| --- | --- | --- | --- |
| # Date: | # Topic # Task Research Study Implementation Circuit analysis | # Resources SLURAQ7.PDF # Components # Additional Notes The circuit uses a half bridge rectifier. Diode # Results |  |
| # Date: 31/05/2024 | # Topic Schematic drawing # Task Research Study Implementation Circuit analysis | # Resources UCC28910 ¦ Buy TI Parts ¦ TI.com # Components UCC28910DR # Additional Notes # Results |  |
| # Date: 31/05/2024 | # Topic Schematic drawing # Task Research Study Implementation Circuit analysis | # Components ## Safety capacitors: A safety capacitor is a special type of capacitor designed to improve the safety of electronic circuits, particularly those dealing with high voltage or AC (alternating current). They primarily address two issues: - Transient voltage spikes: These are sudden bursts of voltage that can damage circuits. Safety capacitors act like a surge protector, diverting excess energy to ground and protecting the device. - Electromagnetic interference (EMI): This is the emission of electromagnetic waves by electronic devices, which can disrupt the operation of other devices nearby. Safety capacitors can help filter out EMI, ensuring cleaner operation. There are two main types of safety capacitors: - Class X capacitors: These are placed directly across the AC power line and are used for line filtering. They typically fail in an open way, meaning they just stop working, which might cause the device not to function but won't create a shock hazard. - Class Y capacitors: These connect one side of the AC line to the chassis ground. They are crucial for preventing shock hazards and are designed to fail open as well. A failing Y capacitor would mean the EMI filtering wouldn't work, but there wouldn't be a risk of electric shock. Safety capacitors are found in many electronic devices, including power supplies, modems, and even everyday appliances. They play a vital role in ensuring the safe and reliable operation of electronics. |  |
| # Date: 03/06/2024 | # Topic Bypass capacitor # Study | # Resources ChatGPT - Tutor Me # Notes The value of the capacitance in bypass capacitors is important because it determines how effectively the capacitor can filter out noise. Here's how: - Low Capacitance: Smaller capacitors (e.g., in the picofarad (pF) range) are better at filtering out high-frequency noise. They react quickly to rapid voltage changes. - High Capacitance: Larger capacitors (e.g., in the microfarad (µF) range) are better at filtering out lower-frequency noise. They provide more energy storage, which helps smooth out larger voltage fluctuations over a longer period. In practice, designers often use a combination of capacitors with different values to cover a wide range of frequencies. For example, they might use both a small capacitor (pF range) and a larger capacitor (µF range) in parallel to filter out both high and low-frequency noise effectively. The material used in bypass capacitors affects their performance, cost, and suitability for different applications. Here are some common types: - Ceramic Capacitors: - Material: Ceramic - Advantages: Low cost, small size, good high-frequency performance, and stability. - Applications: Commonly used for high-frequency noise filtering and general-purpose bypassing in digital circuits. - Film Capacitors: - Material: Various plastic films (polyester, polypropylene, etc.) - Advantages: Good stability, low loss, and excellent performance at high frequencies. - Applications: Used in applications requiring stable capacitance and low dissipation, such as audio circuits and precision analog applications. - Electrolytic Capacitors: - Material: Electrolyte and aluminum or tantalum - Advantages: High capacitance values, suitable for low-frequency filtering. - Applications: Often used in power supply filtering and decoupling where larger capacitance is needed. - Tantalum Capacitors: - Material: Tantalum - Advantages: High capacitance per volume, stable, reliable, and long life. - Applications: Used in space-constrained applications and where high reliability is crucial, such as in aerospace and medical devices. - Polymer Capacitors: - Material: Conductive polymer - Advantages: Low equivalent series resistance (ESR), good high-frequency performance, and long life. - Applications: Used in high-performance applications such as computer motherboards and high-frequency circuits. The choice of material depends on the specific requirements of the circuit, such as the needed capacitance value, frequency range, physical size constraints, and cost considerations. |  |
| # Date: 03/06/2024 | # Topic Voltage sensor # Task Research Study Implementation | # Resources في المختبر: 95- صنع دائرة منافسة (Isolated Voltage Sensor) # Components AMC1200 ROE050S # Additional Notes # Results |  |
| # Date: 04/06/2024 | # Topic # Building buck HV # Task Research Study Implementation | # Notes Built a PCB following # Results - High output ripple - High spike on the one gate - Zener voltage is approx 12V, less than the required 15V needed for IR2110 VCC |  |
| # Date: 07/06/2024 | # Topic # Buck HV # Task Research Study Implementation | # Log: - Testing IR2110 functionality # Notes The IR2110 has malfunctioned due to …connecting ground?? The entire buck got damaged. I am going to rebuild it When building a buck converter and using an IR2110 gate driver to control the switching MOSFET, several factors can lead to MOSFET damage. Here are common reasons for MOSFET failure: - Overvoltage: - Drain-Source Overvoltage (Vds): Exceeding the maximum voltage rating can cause breakdown and permanent damage. - Gate-Source Overvoltage (Vgs): Applying a voltage higher than the maximum Vgs can damage the gate oxide layer, leading to failure. - Overcurrent: - Excessive Drain Current (Id): High currents can cause excessive heating and thermal runaway, leading to failure. - Insufficient Gate Drive Voltage: - The IR2110 requires a proper supply voltage to fully turn on the MOSFET. Insufficient gate drive voltage can result in partial turn-on, leading to high Rds(on) and increased power dissipation. - Poor Gate Drive Design: - Gate Resistor: An incorrect gate resistor value can lead to slow switching times, increasing switching losses and heat. - Parasitic Inductance and Capacitance: These can cause ringing and oscillations, potentially leading to overvoltage and damage. - Inadequate Cooling: - Poor thermal management can cause the MOSFET to overheat. Ensure proper heat sinking and airflow to dissipate heat. - Switching Losses: - Frequent switching at high frequencies can lead to significant switching losses. Proper selection of the MOSFET with low gate charge (Qg) and switching energy (Eon, Eoff) is crucial. - Layout Issues: - Long PCB Traces: High inductance in long PCB traces can cause voltage spikes. - Poor Grounding: A poor ground plane can lead to noise and unstable operation. - Parasitic Elements: - Parasitic Inductance: Inductance in the gate drive circuit can cause ringing and overvoltage conditions. - Parasitic Capacitance: High-frequency operation can cause parasitic capacitances to impact switching performance. - Incorrect Bootstrapping: - If the IR2110 high-side driver uses a bootstrap circuit, ensure the bootstrap capacitor is correctly sized to provide sufficient charge for the high-side MOSFET gate drive. - Fault Conditions: - Short Circuits: A short circuit can cause high current and rapid thermal failure. - Load Transients: Sudden changes in load can induce voltage spikes and stress the MOSFET. To mitigate these issues, consider the following: - Use snubber circuits to dampen voltage spikes. - Ensure the gate drive circuit is properly designed with appropriate resistors and layout considerations. - Use thermal pads, heat sinks, or active cooling to manage heat dissipation. - Select MOSFETs with adequate voltage and current ratings, and appropriate Rds(on) for your application. - Validate your design with simulations and thorough testing to identify and address potential issues early. If you encounter MOSFET failures, providing specific details about your circuit, including schematics, operating conditions, and any observed symptoms, can help in diagnosing the issue more effectively. # Resources # Components # Results IR2110: failed Reasons: unsure, most likely groundco Increase gate resistance. Ask for suggestions Power dissipation. Ask it to show you how for mosfet and diodes Ask it help you size the bootstrap capacitor | - When the system malfunctions. Identify the damaged compon |
| # Date: # 10/06/2024 | # Topic Mosfet Drivers Heat Management # Task Research Study Implementation | # Questions - Driving an inductive load safety concerns. - Freewheeling diodes or MOSFETs with in body diodes - MOSFET selection criteria - Voltage Rating (Vds): - Current Rating (Id): - On-Resistance (Rds(on)): - Lower Rds(on) values result in lower conduction losses, which is crucial for efficient operation. Select MOSFETs with the lowest possible Rds(on) that meet other criteria. - Thermal Management: - Ensure the MOSFET can dissipate heat effectively. Look at the thermal resistance (RθJA, RθJC) and ensure adequate heatsinking or thermal management. - Switching Speed: - Body Diode Performance: - The body diode's reverse recovery time (trr) - reverse recovery charge (Qrr) should be low to handle inductive kickback efficiently. High-performance MOSFETs will have body diodes optimized for fast recovery. - Safe Operating Area (SOA): - Ensure the MOSFET's SOA encompasses the operating conditions, including any transient conditions. - Ruggedness and Reliability: - Look for MOSFETs designed for high-reliability applications, with ratings that ensure they can withstand harsh conditions, including avalanche energy ratings. Ensuring Body Diodes Can Handle Inductive Kickback - Reverse Recovery Characteristics: - Reverse Recovery Time (trr): The time it takes for the diode to switch from conducting to blocking mode. Shorter trr reduces the duration of the reverse current spike. - Reverse Recovery Charge (Qrr): The charge that flows during the reverse recovery time. Lower Qrr reduces the energy dissipated during switching and minimizes voltage spikes. - Thermal Considerations: - Ensure the body diode can handle the power dissipation during the reverse recovery period without exceeding its thermal limits. Check the datasheet for power dissipation ratings and thermal resistance. - Current Rating: - The body diode should have a peak current rating sufficient to handle the inductive kickback without damage. - Paralleling MOSFETs: - In high current applications, paralleling MOSFETs can help distribute the current and reduce stress on individual body diodes. - Snubber Circuits: - Use RC snubber circuits across the MOSFETs to dampen voltage spikes and protect the body diodes from excessive dv/dt. - Gate Drive Management: - Proper gate drive design can ensure smooth switching transitions, reducing the stress on the body diodes. The gate driver should be capable of providing sufficient drive current and voltage to switch the MOSFETs quickly and efficiently. - Power Dissipation Calculation - <<Power Dissipation in MOSFETs.pdf>> - Snubber circuit Design - Decoupling Capacitors # Resources # Components CSD19534Q5A 100 V N-Channel NexFET Power MOSFETs datasheet (ti.com) TIDRN75.PDF TIDA-00365 reference design ¦ TI.com Understanding Buck Power Stages Mode Power Supplies (tij.co.jp) π120u30 Digital Isolator ¦ Tiny Transistors # Additional Notes # Results |  |
| # Date: 12/06/2024 | # Topic # Inverter & Buck control & coding # Task Research Study Implementation | # Notes ## Code Analysis Overview - Initialization - PWM configuration - ADC configuration - PI controller Setting Up Hardware Voltage Divider Circuit Power Supply and Decoupling: - Ensure a stable 3.3V supply for the MCU and reference voltage. - Use decoupling capacitors (e.g., 0.1 µF ceramic capacitor) close to the ADC input pin to filter out high-frequency noise. Setting Up Software - ADC Configuration: Initialize and configure the ADC to read the voltage at VfbV_{\text{fb}}Vfb​. Ensure the ADC is set to use a 3.3V reference. - Voltage Calculation: Convert the ADC reading to the corresponding VoutV_{\text{out}}Vout​ value in software. LV_Buck feedback voltage setting Voltage reference??? # Resources # Components # Results | Inconclusive |
| # Date: 13/06/2024 08:11 | # Topic # Inverter: Human interface # Task Research Study Implementation | # Write a function to increment the frequency of the inverter: # Resources # Components # Additional Notes # Results |  |
| # Date: 14/06/2024 | # Dead Time Generation | # Resources # Components # Additional Notes # Results Simulation Increased the Deadtime to 200us, since the inverter frequency low Used 100 nF capacitor |  |
| # Date: 17/06/2024 | # Topic Debugging Bias circuit (3.3V bias voltage) What went wrong with LM2576T? # Task Research Study Implementation | # Resources LM2576-5 buck converter overheating advice. - Page 1 (eevblog.com) # Components # Notes LM2576T heats up, with very little load Once connected, it exploded What's the total load? IR2110 74HC14N STM32F10 Does the total load exceed the rated IC current? ---------------------- Is the MCU drawing too much current? Is the voltage divider circuit drawing too much current? # Results Wrong wiring in schematic |  |
| # Date: 19/06/2024 | # Topic Buck converter Debugging # Task Research Study Implementation | # Using a safety capacitor with Buck - EMI suppression capacitors are not used for power supply filtration. ## Buck Converter Capacitor considerations - Capacitance Value - Capacitor Type: Ceramic (X7R or X5R) with low ESR and ESL What’s the voltage and current passing through the gate diode # Resources # Components # Additional Notes In the Initial Design I used the wrong Value of Capacitor 19/06/2024 10:42 The buck converter operates normally with low resistive loads, but does not when it's hooked to an inductive load. The mosfet switching is inconsistent. Tends to skip switching cycles. The Buck lacks decoupling capacitors Proper isolation and decoupling between the buck converter and the inverter are crucial to prevent interference and ensure stable operation. Let's delve into how to achieve this: GPT /* 1. Decoupling Capacitors Decoupling capacitors are used to filter out high-frequency noise and provide a local reservoir of charge to stabilize voltage levels. Placement: Place decoupling capacitors as close as possible to the power supply pins of both the buck converter and the inverter. Types and Values: Use a combination of different capacitors to cover a broad frequency range. Common values include: Ceramic Capacitors: 100 nF, 1 µF, 10 µF Electrolytic Capacitors: 100 µF or higher for bulk decoupling 2. Grounding Scheme A proper grounding scheme is essential to avoid ground loops and ensure a clean signal ground reference. Star Grounding: Use a single point (star ground) where all grounds from the buck converter and the inverter are connected. This minimizes ground loop issues. Separate Ground Planes: If possible, use separate ground planes for the power and signal grounds and connect them at a single point. 3. Filtering Add filters to suppress high-frequency noise and harmonics that might be coupling between the buck converter and the inverter. LC Filters: Use LC filters at the output of the buck converter and the input of the inverter to attenuate high-frequency noise. Ferrite Beads: Place ferrite beads in series with power lines to further reduce high-frequency noise. 4. Shielding and Layout Proper PCB layout and physical separation can significantly reduce interference. Physical Separation: Keep the buck converter and inverter circuits physically separated on the PCB to minimize coupling. Shielding: Use shielding around critical components and signal traces if necessary. 5. Snubber Circuits Snubber circuits can help dampen oscillations and protect components from voltage spikes. RC Snubbers: Place RC snubbers across the switching MOSFETs in both the buck converter and the inverter to reduce switching noise and ringing. Immediate Steps to Implement Isolation and Decoupling Add Decoupling Capacitors: Place a 100 nF ceramic capacitor and a 10 µF ceramic capacitor close to the power supply pins of the buck converter. Repeat the same for the inverter. Check Grounding: Ensure all grounds from both stages meet at a single point. Use a star grounding scheme. Implement LC Filters: Add an LC filter at the output of the buck converter (e.g., 10 µH inductor and 10 µF capacitor). Add another LC filter at the input of the inverter. Test with Load: Test the buck converter with both resistive and inductive loads. Verify stable DC output before connecting to the inverter. */ # Filtering Pi Filter - SoCs - a high pass filter or a low pass filter - 3rd order in nature and provide strong rolloff above the cutoff (~20 dB/decade). - We design for the ripple factor - Ripple factor = (V_max -V_min)/V_output - Ripple factor depends on the load - The power remains constant, but the other variables change; that is the voltage, current and the current varies with varying the load ## Regulated power supplies - Switched - Linear - The value of the ripple factor depends on the Load R. For an unregulated power supply this is the load connected at the output. - Ferrite choke as the inductor. ESR, Series resistor on the ferrite choke to dampen the ringing in these filters. - They should be used with switched mode power supplies. - Impedance matching: Matching the source resistance to the load - Two ways to designing a PI filer: - the cutoff frequency # Results Pulsating output. |  |
| # Date: | # Topic # Power Supply Design # Task Research Study Implementation | # Resources tidm559.pdf PMP30446 reference design ¦ TI.com # Components UCC28051DR UCC21220AD UCC2871x Constant-Voltage, Constant-Current Controller With Primary-Side Regulation datasheet (Rev. C) # Additional Notes # Results |  |
| # Date: 21/06/2024 | # Topic # Power Supply Design # Task Research Study Implementation | # Resources Power supplies TI Reference Designs Library # PMP10110 Rev_D Schematic.PDF PMP10110 reference design ¦ TI.com *PMP20122_AssemblyRelease (ti.com) Buck converter # Components # Additional Notes flyback power supply controller Can I find a reference design that can give me exactly 15V? Yes Can I buy the kind of Transformer that I need? https://www.mouser.vn/datasheet/2/447/P846-2902981.pdf # Results Input Voltage is: 33-57v https://www.mouser.vn/ProductDetail/Wurth-Elektronik/750343068?qs=qSfuJ%252Bfl%2Fd5S1vXLpvktrw%3D%3D ------------------------------------------------------------------------------------------------------------------ # Flyback Transformers No; Description; Primary Voltage Rating; Secondary Voltage Rating; Power; Freq kHz. 750312495; 750 uH 21.6 W; 85 VAC to 265 VAC; 12 V; 21.6 W; 100. 750311771; 400 uH 45 W; 157 V to 629 VAC; 15 V; 45 W; 60 - 400. 750311715; 224 uH 30 W; 94 VAC to 940 VAC; 15 V; 30 W. |  |
|  | # Power Supply Design 350V - 15V-3V ----------------------------- - Smallest packages I am going for are 0603 - Buying according to stock | # References https://www.ti.com/lit/df/tidrak4/tidrak4.pdf?ts=1719046893456 CMC is used when AC/DC L2: 11mH, 250V BPWX 0.5-14-33 BIAS Power ¦ Mouser Vietnam PMP41037_ControlE1(001)_Sch.PDF Power Line inductors Cutoff frequency Current # Result 15V 3.3V Using a buck converter Isolated 3.3V SN6501: 350 mA TPSM84209 data sheet, product information and support ¦ TI.com Ca be used to get 3.3V with minimal componets ------------------------------------------------------------ Gonna use the IC I have lm2576-3.3V LM2576T-ADJ/NOPB Texas Instruments ¦ Mouser Việt Nam Passive componets Inductor 470uH 1A YPRH1209-471M ¦ YJYCOIN ¦ Power Inductors ¦ JLCPCB 1N5822 ¦ MDD(Microdiode Electronics) ¦ Schottky Barrier Diodes (SBD) ¦ JLCPCB C_in CC0603KRX7R9BB104 ¦ YAGEO ¦ Multilayer Ceramic Capacitors MLCC - SMD/SMT ¦ JLCPCB Bulk cap RT1A102M0810 ¦ ROQANG ¦ Aluminum Electrolytic Capacitors - SMD ¦ JLCPCB # AC Filter w Components 37013150000 ¦ Littelfuse ¦ Fuses ¦ JLCPCB TIDA-050074 Schematic # Footprint check 744822301 Datasheet WE-CMB Common Mode Power Line Choke (we-online.com) |  |
| 24/06/2024 | Bulk Capacitor Selection | # Bulk Capacitor Calculation Cap >150uF |  |
|  | Voltage Sensing Circuit Design | # Voltage Sensing Circuit ## Resources AMC1311x High-Impedance, 2-V Input, Reinforced Isolated Amplifiers datasheet (Rev. C) (ti.com) ----------------------------------------------------------------------------------------------------------------------- في المختبر: 95- صنع دائرة منافسة (Isolated Voltage Sensor) ## Video Notes LV25-p sensor - High V in - low V out - +, - power supply - Ac output voltage -> voltage shifting AMC12XX isolated amplifiers features - High in low out - 250mV in - Fixed gain - Frequency bandwidths - 8x gain - Level shift - Output is shifted according to the VDD - If 5VDD, output is 2.55V - If 3.3VDD, output is 1.29V Isolated Power supply - 5V in, 5V out Schematic - 10X 1X amplification option - 3.3V, 5V option - 3.3V regulator to step down 5 - 3.3V- switch seleciton - Big componets for testing ---------------------------------------------------------------------------------------------------------------------- TIDA-010054 reference design ¦ TI.com ----------------------------------------------------------------------------------------------------------------------------- # AMC1113 Datasheet Missing high side indication 100 mW Keep input voltage below 2V Output common-mode voltage VCMout 1.44 If the input voltage exceeds the abs max rating, the current must be limited to the max value Current shouldn't exceed 10mA, V The maximum continuous input current for the IN pin, as specified in the Absolute Maximum Ratings section, is ±10 mA. ---------------------------------------------------------------------------------------------------------------------------------------------------------------------- # Circuit Design # Multilayer Ceramic Capacitors MLCC - SMD/SMT ¦ Capacitors ¦ JLCPCB Assembly Parts Library 16V 1uF X5R ±10% 0603 CL05B104KO5NNN 100nF 0603 X7R CL10B105KA8NNNC 1uF 0603 X7R CC0603KRX7R9BB101 50V 100pF 0603 X7R Resistors C245367 125mW Thick Film Resistors 150V 665kΩ 0805 0603WAF2002T5E 75V 20kohm We have 6 resistors Power per 1 resistor is 6mW # Side Notes - "amplifiers" doesn’t strictly means amplifying it also indicated attenuation - Shunt resistors - Planer transformer - PGND, SGND, GND - Dual, precision, zero-crossover, - Low ESR capacitors. X5R, X7R - MLCC_2014_1_표지 (lcsc.com) solid supplier |  |
|  | Buck Design - SWITCHING ELEMENT SELECITON - Inductor Selection - Inductor layout - Diode Selection | # Selection Criteria V > 350 I > 3 # MOSFET Selection No; V_DS; I_D; R(on) mΩ; Gate Charge (nC); Tr nS; t_d(on); t_d(off); t_f; f(Mhz). IRf730; 400; 5.5; 1000; 38; 15; 10; 38; 14; 12. IPA60R280CFD7XKSA1; 600; 6 A; 237; 18; 14; 17; 53; 9; 10. IMBG65R260M1H; 650; 6 A; 346; 22; 5; 5.3; 7.7; 13; 32. IRF730 is sufficient for our current application. Switching losses is very high. Switching frequency is sufficient for this applicaiotn ---------------------------------------------------------------------------------------------------------------------------- # Inductor Selection Board Layout application Notes for switching regulators https://www.mouser.vn/ProductDetail/Schurter/DSO1-48-0001?qs=A5k8bsa1loCxJozGZre6sQ%3D%3D Model; Current; Inductance. 2319-V-RC; 3A; 390 uH. HCTI-390-5.0; 5A; 390 uH. -------------------------------------------------------------------------------------------------------------- # Diode Selection RHRP860 Diode meets the criteria -------------------------------------------------------------------------------------------------------------- How to design the power supply circuitry to garaurntee proper isolation? 3.3V power supply to MCU, AGND 15V RO153.3S ---------------------------------------------------------------------------------------------------- Protection Circuit Fuse Varistor Selection Model; Continuous DC voltage; Varistor Voltage. 100 V; >100V. V120MA2S; 102; 120. MOV-20D121K; 100; 120. V120ZA20P; 102; 108. ---------------------------------------------------------------------------------------------------- # Bulk Capacitor selection Criteria: size voltage and capacitance # Gate Diode & Gate Resistor Voltage 15V, fast switching, fast recovery # Bootstrap Diode https://www.mouser.vn/ProductDetail/Diodes-Incorporated/MURS160-13-F?qs=hiZRsJxw0h10uqIKGq7eFw%3D%3D |  |
|  | Circuit Review | Power Supply Design V_in V_in min 250 V_in max 350 |  |
| 01/07/2024 |  |  |  |
|  | - Bleed Resistor Calculations - Set Selection Criteria - Select IC | # Bleed Resistor Selection Calculations # Criteria T= 30S C=100uF R= 300kohm P>1W Resistor; Resistance; Power rating; Voltage Rating. ERJ-UP6D3003V Panasonic ¦ Mouser Vietnam; 300k; 500 mW; 400 V. ERJ-UP8F3003V Panasonic ¦ Mouser Vietnam; 300 kOhms; 660 mW; 500 V. RN65D3003FB14 Vishay / Dale ¦ Mouser Vietnam; 300 kOhms; 500 mW (1/2 W); 350 V. H4P300KFZA; 300 kOhms; 1 W; 500 V. PR02000203003JR500 Vishay / BC Components ¦ Mouser Vietnam; 300 kOhms; 2 W; 500 V. ------------------------------------------------------------------------------------ Select CRCW251260K4FKTG Vishay / Dale ¦ Mouser Vietnam ------------------------------------------------------------------------------------ |  |
| 02/07/2024 | Gate Diode Selection | NSR0320MW2T1G onsemi ¦ Mouser Malaysia Gate Resistor |  |
|  | # Prototype 1 | # DC/DC Calculations ------------------------------------------------------------- # AC/DC & Power Supply Board |  |
| 05/07/2024 |  | # High Voltage deisng considerations Understanding High Voltage PCB Materials # FR4 material Fiber Resin Weave of the fiber Higher resign material More space less tight weave Curing agent Cphenolic DICY CTI PLC Deligniate High frequency Vs high voltage CAF resistance CTI Value Material that has an approperiate CTI CAF failue You should build your own library Size constraints Layout is important Ground Pour Copper pour Pour over the trace Simulate a bus bar? C3 coating Spacing /Clearance Trace width PCB Fabrication, Assembly, and Components ¦ Sierra Circuits (protoexpress.com) Trace Width Calculator ¦ AdvancedPCB Decided to keep the design onto one layer for simplicity, the board is a 4 layer board. There's redundancy |  |
|  | Control Circuit | Crystal https://jlcpcb.com/partdetail/Yxc-X322525MOB4SI/C9006 https://www.mouser.vn/ProductDetail/RECOM-Power/R1S-3.33.3-H?qs=XF8hdbuHJAVie79Sg7ec0g%3D%3D |  |
|  |  | # Inductor Selection Inductors Selection Criteria, in case you wanna buy it locally Current > 3A >250 uH Designed for DC converter Part Number; Saturation Current; Current; Inuductance. 2218-H-RC Bourns ¦ Mouser Vietnam; 3.3 A; 330 uH. PE-53117NL Pulse Electronics ¦ Mouser Vietnam; 3 A; 330 uH. SHBC24W-2R1A0311V KEMET ¦ Mouser Vietnam; 15 A; 311 uH. 2100HT-221-V-RC Bourns ¦ Mouser Vietnam; 3 A; 220 uH. 2300HT-331-V-RC J.W. Miller ¦ Mouser Vietnam; 5A; 330 uH. AIRD-03-331K ABRACON ¦ Mouser Vietnam; 4.5 A; 330 uH. 1433445C Murata Power Solutions ¦ Mouser Vietnam; 4.5 A; 330 uH. PCV-2-105-02L Coilcraft ¦ Mouser Vietnam; 2.3 A; 1 mH. ------------------------------------------------------------------------------------------------------------------------------------------------------------------------------- # PCB layout Zero ohm resistors to s Start with the most important then place the secondary componets Boot resistor placement doesn’t necessarily need to be close to the pin Keep the ics with their componets close together. IMUss ------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------ # Checking schematic |  |
|  | PCB Design Revision | - Traces, no sharp turns - Traces carrying high currents should be sized appoperiately - Traces carrying critical analog signals, should not run parallel to traces carrying fast changing signals. Due to noise issues - Traces connecting inductors should not be wider than necessary - Power plane on an internal layer - 10 uF bulk capacitors, 100nF - 1 decoupling cap for every power pin - Delay lines for clock lines. Trance length equalization - Timed relationship between multiple signals. e.g. routing high speed signal to multiple chips - Inductors shouldn't be plaved too closely together - Inductors shouldn't be plaved too close to large metalic objects. - Power components shouldn't be plaved colse to each other - The components associated with a switching regulator should be localized to a section of the PCB, far away from sections handing analog signals - The AC section should be localized to one section of the board. - A physical barrier seperating the ac from the rest of the borad. - PCB solt seperating the 2 sections - 4 layer boards. The Internal 2 layers should be the supply and the ground plane. - If the design contains digital and analog sections. The ground should be split and only joined at one point. The power supply negative. - Gound should connect at one point if we are using 2 layer boards. - Design for manufacturablility and design for testing - Landing patterns. Print it out into 1 to 1 to see if matching. Order samples. - Switching regulator layout rules - Top 5 Beginner PCB Design Mistakes (and how to fix them) - Trace spacing > double check the DRE - Keep clearcing wide. - Cross coupling ? - Keep clearance 3X dilectric thickness of between the layers - Keep parallel sigments between traces as short as possible - High impedance trace is very sensitive to noise, to minimize any coupling to it, we keep the trace as wide as it needs to be - For vias, 0.7, 0.3 - 0.5, 0.3 - Benefit of the capacitance of the plane - Any signal trace should run adjacent to a reference (ground) or power plane - Copper Thickness Deep Dive ¦ PCB Manufacturing |  |
|  | Preorder Review | AC/DC Bleed Resistor updated CMC wrong footprint The difference? AMC requires 5V Modifying the power supply input voltage |  |
|  | Controller Design | Chat GPT PID Control Pros: - Wide Usage: Well-understood and widely used in industry. - Good for Steady-State Error Reduction: Can be tuned to reduce steady-state error effectively. - Versatile: Can handle various types of systems with appropriate tuning. Cons: - Complex Tuning: Requires careful tuning of three parameters (Proportional, Integral, Derivative). - Can be Overkill: Might be more complex than necessary if your system doesn't require high precision. - Response Time: Not the fastest response time compared to simpler control methods. 2. Hysteresis Control Pros: - Simple Implementation: Easy to implement in a microcontroller. - Fast Response: Very fast response time. - Good for Switching Converters: Works well with switching power converters. Cons: - Variable Switching Frequency: Can cause electromagnetic interference due to the variable switching frequency. - Noise Sensitivity: Sensitive to noise, which can affect performance. 3. Bang-Bang Control Pros: - Simplicity: Very easy to implement. - Fast Response: Provides a very quick response. - No Tuning Required: No need for parameter tuning. Cons: - Overshoot and Undershoot: Can cause significant overshoot and undershoot. - Stress on Components: Can stress the power electronics due to frequent switching. 4. Model Predictive Control (MPC) Pros: - Advanced Control: Can predict future behavior and adjust accordingly. - Optimal Control: Often results in optimal control performance. Cons: - Complex Implementation: More complex to implement and requires significant computational resources. - Not Always Necessary: Might be overkill for simpler systems without stringent performance requirements. Recommendation For your application, I recommend starting with PID Control due to its balance between performance and complexity. It's widely used, well-documented, and provides good performance for a variety of systems. You can use tools like MATLAB or specific libraries to help with tuning the PID parameters. If simplicity and fast response are more critical, Hysteresis Control might be a good alternative, especially for switching converters like a buck converter. However, be aware of the potential issues with variable switching frequency. Would you like guidance on implementing any of these control algorithms in your microcontroller? ------------------------------------------------------------------------------------------------------------------------------------------------------------------------ # PID and Hysteresis Control seem to be a good fit Investigate them further # PID Trial and Error Test ## P Stable ## PI Unstable ### PID How to collect data from HW to PID Tune? |  |
|  |  | PID Controller Implementation in Software - Phil's Lab #6 A real control system - how to start designing To design a buck converter using a PID controller with a “Hardware in the Loop” approach, you can follow these generalized steps based on the video: - Understand the System: - Identify the key components: buck converter, PID controller, sensors, and actuators. - Understand the physics and dynamics of the buck converter, including input voltage, output voltage, and load conditions. - Set Up the Hardware: - Use a microcontroller (e.g., Arduino) to manage the sensors and actuators. - Connect the buck converter to the microcontroller and set up the necessary sensors to measure output voltage and current. - Initial Testing: - Perform open-loop tests to understand the system’s response to different input conditions. - Record the step response of the buck converter to a given input voltage. - Develop a Mathematical Model: - Use the recorded step response to create a first-order plus dead time (FOPDT) model of the buck converter. - Identify the gain, time constant, and delay of the system. - Design the PID Controller: - Implement a basic PID controller in software (e.g., MATLAB or Python). - Use the FOPDT model to tune the PID parameters (P, I, D) for optimal performance. - Hardware in the Loop Testing: - Run the PID controller on your main computer and send commands to the microcontroller. - Monitor the buck converter’s response and adjust the PID parameters as needed. - Iterative Refinement: - Continuously test and refine the PID controller using the hardware setup. - Address any discrepancies between the model and the actual hardware performance. - Final Implementation: - Once satisfied with the controller’s performance, implement the PID controller directly on the microcontroller. - Conduct final tests to ensure stability and performance under various operating conditions. By following these steps, you can effectively design and implement a PID controller for a buck converter using a “Hardware in the Loop” approach. |  |
|  |  | How to Model a Synchronous Buck Converter Linearizing Simulink Models https://www.youtube.com/watch?v=VQ3Er89Mwa8&t=1s&ab_channel=katkimshow |  |
|  | System Modelling and Linearization | DC-DC Converter Control: Modeling https://www.youtube.com/watch?v=aa5M1HiM7Ac&t=86s&ab_channel=katkimshow |  |
|  |  | -------------------------------------------------------------------------------------------------------------------------------------------------------------------------------- The PID parameters are set. There's an overshoot when setting the voltage value. That has to be addressed with a damper. |  |
|  | System Identification | https://www.mathworks.com/help/simulink/slref/output-simulation-data-with-blocks.html |  |
|  | PID Implementation in SW | PID Controller Implementation in Software - Phil's Lab #6 -------------------------------------------------------------------------------------------------------------------- |  |
|  |  | SystemCoreClock = 64 MHz Prescaler = 6400 -1 => TIM3 = 10 KHz, ARR = 10 KHz /10,000-1 => 1 second interrupt |  |
|  | Interrupt config | https://community.st.com/t5/stm32-mcus/how-to-generate-a-one-second-interrupt-using-an-stm32-timer/ta-p/49858 Start timer Start interrupt Define an interrupt global fucntion void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim); float MeasuredVoltage = ReadFBVoltage(&hadc1); float normalizedVoltage = MeasuredVoltage * (R1 + R2) / R2; float controlSignal = PIDController_Update(&pid, DESIRED_VOLTAGE, normalizedVoltage); setDutyCycle(&htim1, TIM_CHANNEL_1, controlSignal); // MCU system clock // uint32_t hclk_freq = HAL_RCC_GetHCLKFreq(); |  |
| 23/07/2024 | Interfacing | The MCU should have 2 MCUs. - Interfacing with LCD - buck PWM generation What communication protocol to use? - I2C is for streams of data - CAN for signals Steps - User Interface circuit - Draw a schematic & Define requirements - Design circuit - BOM - Communication I want to design a user interface circuit for the VFD. Requirements - Receive input voltage, input frequency from user - Display curret voltage and frequency setting to the user |  |
| 26/07/2024 | SMD Soldering and Assembly Requirements | Hot Air Gun if possible High quality Solder paste High quality solder <0.4 Flux Alchohol Cotton swabs Very thin tweezers Desoldering copper braid |  |
| 30/07/2024 | Notes about the PCB assembly & BOM | Inductor missing, we improvised and used a smaller inductor value 200 uH, 3A and no datasheet |  |
| 30/07/2024 30/07/2024 09:03 | LCD Interface How to | Rotary Encoder (KY-040) Debounce Circuit Tutorial |  |
|  |  | <<rectifiedData.xlsx>> %%Time specifications: Fs = 8000; % samples per second dt = 1/Fs; % seconds per sample StopTime = 0.02; % seconds t = (0:dt:StopTime-dt)'; % seconds %%Sine wave: Fc = 50; % hertz x = 340*sin(2*pi*Fc*t); % Plot the signal versus time: figure; plot(X,Y); xlabel('time (in seconds)'); title('Signal versus Time'); zoom xon; hold on % Define the number of samples samples = 64; % Generate the x-axis values samples = linspace(0, StopTime, samples); % Generate the sinusoidal signal y = 340*sin(2 * pi * samples*Fc); % Plot the sinusoidal signal stem(X, Y, 'b', 'LineWidth', 0.1); % Blue lines plot(X, Y, 'bo', 'LineWidth', 1); % Blue circle markers |  |
|  | Design review | I have divided the program into 2 parts; User Interface and control scheme. User Interface Componets: LCD, Rotary Encoder Function: The user input the desire voltage and frequency using the encoder and it gets displayed on the LCD Control Scheme Componets: 2 parts; Buck and Inverter Function: It send the PWM signal to the buck switching element, and send the inveter H-bridge to control the frequency. When the system starts, by default it starts at 50V and 20Hz. The user can adjust these values by switching between 2 modes. Frequency and Voltage The user has to first input the desried voltage value then it gets displayed on the LCD. The display displayes 2 values; Desired Voltage and Actual Voltage The display is updated every one second. Prepherals Timers Timers1: Buck PWM Timer2: Inverter PWM Timer3: Updated The LCD every one second ADCs ADC1 & ADC2: Voltage Sensor ------------------------------------------- |  |
|  |  | Modular and state management code Why Use a State Machine? A state machine helps in: - Organizing Code: It makes the code easier to read and maintain by clearly defining states and transitions. - Managing Complexity: It simplifies the management of different modes and their transitions. - Avoiding Bugs: By clearly defining transitions, it reduces the risk of unexpected behavior. |  |
|  | Stm32 cofiguration | # ADC Configuration How to use adc 1 channel 1 and 2 to read the data? |  |
|  |  | # Buck Calibration Objective: determine the output voltage to the feedback voltage ratio Provide the buck with a 50% duty cycle to determine the Procedure Provide the buck with varying duty cycle and measure the output voltage and the feeback voltage to determine the output. ------------------------------------ |  |
|  | Testing and verification | Buck Test In this step I am testing the functionality of the buck converter measuring the input and the output voltages and the feedback signal -------------------------------- Test the regulation mechanism The ISR routine takes too long. |  |
|  | Using Usart | using usart I want to establish communication between 2 MCUs, where one would sends data every one second and the other receives it. one mcu is used for control and the other is for interface. the protocol is uart. First the user inputs the desired voltage and frequency, the data get's transferred to the Control interface the second part of the I need a way of determing when the desired data has been received If the a certain character is received, proceed |  |

Total ISR duration 61us

How long does each segement take???

| Segment | function | Cycles | Time (us) | Optimized code | Cycles After optimization | Time (us) After optimization |
| --- | --- | --- | --- | --- | --- | --- |
| ADC1_SetChannel(ADC_CHANNEL_0, ADC_REGULAR_RANK_1); | Setting the first channel | 275 | 3.7 |  | 46 | 0.64 |
| measuredVoltageCh1 = ReadFBVoltage(&hadc1); | Reading and converting the voltage | 1779 | 24.708334 |  |  | 6.9 |
| ADC1_SetChannel(ADC_CHANNEL_1, ADC_REGULAR_RANK_1); | Setting 2nd channel | 232 | 3.22222209 |  |  | 0.64 |
| measuredVoltageCh2 = ReadFBVoltage(&hadc1); | Measuring the second channel | 1780 | 24.7222214 |  |  | 6.9 |
| differentialVoltage = measuredVoltageCh1 - measuredVoltageCh2; normalizedVoltage = differentialVoltage * VoToVfb; ccm_val = __HAL_TIM_GET_COMPARE(&htim1, TIM_CHANNEL_1); if (normalizedVoltage < DESIRED_VOLTAGE) { ccm_val += 1; } else if (normalizedVoltage > DESIRED_VOLTAGE) { ccm_val -= 1; } __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, ccm_val); | Calculating the measured voltage | 390 | 5.41666651 |  |  | 5.26388884 |

Sum: 61us

Optimized :19us

| Tim1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Tim2 |  |  |  |  |
| DMA | Mode: circular, word, word, Disable global interrupt |  |  |  |